Global Product Strategy (GPS) Safety Summary

Phenthoate Technical

This GPS Safety Summary is a high-level summary intended to provide the general public with an overview of product safety information on this chemical substance. It is not intended to provide emergency response, medical or treatment information, nor to provide an overview of all safety and health information. This summary is not intended to replace the Safety Data Sheet. For detailed guidance on the use or regulatory status of this substance, please consult the Safety Data Sheet and the Product Stewardship Bulletin (PSB).

Chemical Identity

Name: Phenthoate Technical
Chemical name (IUPAC): O,O-dimethyl O-(3-methyl-4-(methylthio)phenyl) phosphorothioate
CAS Number: 2597-03-7
EC Number: 219-997-0
Molecular Formula: C12H17O4PS2
Molecular Weight: 320.36 g/mol

Uses and Applications

Phenthoate Technical is an organophosphorus insecticide used in agriculture for the control of a wide range of insect pests. It is effective against sucking and chewing insects such as aphids, thrips, leafhoppers, and caterpillars on crops including fruits, vegetables, rice, and cotton.

Phenthoate acts primarily by inhibiting acetylcholinesterase (AChE) in insects, resulting in disruption of nerve impulse transmission and eventual pest mortality. It is commonly used in formulated products and integrated pest management (IPM) programs to ensure effective crop protection.

Due to its toxicity profile, Phenthoate usage is strictly regulated. Proper application practices, adherence to recommended dosages, and use of appropriate personal protective equipment (PPE) are essential to minimize risks to human health and the environment.

Physical / Chemical Properties

Phenthoate Technical is a pale yellowish liquid with aromatic odor. Non-flammable under normal storage conditions.

Health Effects

Phenthoate Technical is classified as hazardous under GHS due to acute toxicity and neurotoxic effects.

The table below gives an overview of the health effects assessment results for Phenthoate.

| Effect Assessment | Result |
| --- | --- |
| Acute Toxicity Oral / inhalation / dermal | Acutely toxic via the oral, and inhalation routes of exposure. |
| Irritation / corrosion Skin / eye/ respiratory tract | May cause skin and eye irritation. |
| Sensitization | Mild Sensitizer |
| Toxicity after repeated exposure Oral / inhalation / dermal | Repeated exposure to Phenthoate through oral, inhalation, or dermal routes can cause chronic toxicity. |
| Genotoxicity / Mutagenicity | No conclusive evidence. |
| Toxicity for reproduction | Effects observed only at high exposure levels in animal studies. |

Environmental Effects

Phenthoate Technical is highly toxic to aquatic organisms and may cause long-lasting effects in aquatic environments.

The table below gives an overview of the environmental assessment results for Phenthoate.

| Effect Assessment | Result |
| --- | --- |
| Aquatic Toxicity | Moderately toxic to aquatic life under the Globally Harmonized System (GHS |

| Fate and behaviour | Result |
| --- | --- |
| Biodegradation | Moderately biodegradable |
| Bioaccumulation potential | Low to moderate |
| PBT / vPvB conclusion | Not classified. |

PBT = Persistent, Bio-accumulative and Toxic in the environment.

vPvB = very Persistent and very Bio-accumulative in the environment.

Exposure

Human health

| Respiratory protection | : | Wear a respirator with a particle filter mask (protection factor 20) conforming to European Norm EN149FFP3 or EN140P3 or equivalent. Respiratory protection should only be used to control residual risk of short duration activities, when all reasonably practicable steps have been taken to reduce exposure at source e.g. containment and/or local extract ventilation. Always follow respirator manufacturer's instructions regarding wearing and maintenance |
| --- | --- | --- |
| Hand Protection Eye protection | : : | Please observe the instructions regarding permeability and breakthrough time which are provided by the supplier of the gloves. Also take into consideration the specific local conditions under which the product is used, such as the danger of cuts, abrasion, and the contact time. Wash gloves when contaminated. Dispose of when contaminated inside, when perforated or when contamination on the outside cannot be removed. Wash hands frequently and always before eating, drinking, smoking or using the toilet. Use equipment for eye protection tested and approved under appropriate government standards such as EN 166(EU). Safety glasses |
| Skin and body protection | : | Wear standard coveralls and Category 3 Type 4 suit. If there is a risk of significant exposure, consider a higher protective type of suit. Wear two layers of clothing wherever possible. Polyester/cotton or cotton overalls should be worn under chemical protection suit and should be professionally laundered frequently. |

Environment

Discharge Prevention: For Phenthoate Technical, environmental exposure should be strictly avoided to minimize the risk of contamination of soil, surface water, and groundwater.

Leakage or Spillage Protocol: In the event of leakage or spillage, stop the source of release immediately if it can be done safely. Implement prompt containment measures to prevent the spread of the material and reduce environmental impact.

Contaminated Wash Water: All contaminated wash water resulting from cleanup operations must be collected and disposed of in accordance with applicable regulations. Discharge into drains, soil, or natural water bodies must be strictly avoided.

Risk Management Measures

- For detailed guidance on the use of Phenthoate technical, the Safety Data Sheet and the Product Safety Bulletin should be consulted.

- Phenthoate should be handled only by knowledgeable and trained personnel.

Flammability

Phenthoate is non-flammable under normal conditions

Human Health

Ventilation & PPE: Ensure adequate ventilation when handling Phenthoate Technical to prevent inhalation exposure. Always wear suitable personal protective equipment, including chemical-resistant gloves, protective goggles or safety glasses, and appropriate protective clothing. Use a face shield or respiratory protection where there is a risk of splashing, mist formation, or inhalation. Ensure all PPE is clean, in good condition, and used correctly.

Hygiene: Eating, drinking, or smoking must be prohibited in areas where Phenthoate Technical is handled or stored. After handling, wash hands and exposed skin thoroughly with soap and water. In case of eye contact, rinse immediately with clean water for at least 15 minutes and seek medical advice. If skin contact occurs, wash thoroughly with soap and water and remove contaminated clothing promptly. Ensure no chemical residue remains on the skin before reuse of clothing.

Transfer & Maintenance: During transfer, loading, or maintenance operations, ensure that transfer lines are cleared and systems are flushed or drained into a closed and labelled container for proper reuse or disposal. Verify that all equipment is securely closed before opening any system. Conduct regular inspection and maintenance of transfer systems to minimize the risk of leaks or accidental exposure. Follow approved decontamination procedures for all equipment after use.

Environmental

In case of accidental release or spill, do not allow the product to enter sewers, surface or ground water.

Regulatory Information / Classification and Labelling

REACH - Candidate List of Substances of Very High 	: Not Applicable

Concern for Authorization (Article 59).
Regulation (EC) No 1005/2009 on substances that 	: Not Applicable

deplete the ozone layer
Regulation (EU) 2019/1021 on persistent organic 	: Not Applicable

pollutants (recast)
REACH - List of substances subject to authorization 	: Not Applicable

(Annex XIV)

| Seveso III: Directive 2012/18/EU of the European Parliament and of the Council on the control of major-accident hazards involving dangerous substances. | E1 | : Environmental Hazards |
| --- | --- | --- |

Conclusion Statements

# Phenthoate Technical is an effective organophosphate insecticide widely used for the control of a broad spectrum of insect pests in agriculture. However, due to its high toxicity, it must be handled, applied, and disposed of strictly in accordance with recommended safety guidelines and regulatory requirements to reduce risks to human health, non-target organisms, and the environment. Incorporating Phenthoate into integrated pest management (IPM) programs, with appropriate rotation and controlled use, is essential to maintain its efficacy and to minimize the development of pest resistance.

# Phenthoate Technical is classified as toxic to aquatic life under the Globally Harmonized System (GHS), as it may cause adverse effects in aquatic organisms and ecosystems. Adequate measures should be taken to prevent contamination of water bodies during handling, application, and disposal.

Contact Information within Company

For further information on this product in general, please consult the Coromandel International limited corporate website (https://www.coromandel.biz/)

Date of issue: 22nd December 2025.

# Disclaimer

The above information is intended to give general health and safety guidance on the storage and transport of the substance or product to which it relates. The requirement or recommendation of any relevant site or working procedure, system or policy in force or arising from any risk assessment involving the substance or product should take precedence over any of the guidance contained in this safety data sheet where there is a difference in the information given. The information provided in this safety data sheet is accurate at the date of publication and will be updated as and when appropriate. No liability will be accepted for any injury loss or damage resulting from any failure to take account of information or advice contained in this safety data sheet.

End of GPS Sheet